2015 HONDA CIVIC

Maintenance sheet — what it needs, when, and what it should cost

Covers the 9th-generation Civic (2012–2015). Primary case is the 1.8L four (R18Z1) in LX / SE / EX / EX-L. Si (2.4L) and Hybrid differences are noted where they matter.

This car is 11 model years old. That drives most of what follows: the age-based items on page 2 are the ones most likely overdue.

# Quick reference

| Item | Answer |
| --- | --- |
| Engine oil | 0W-20 full synthetic — 3.9 qt with filter (1.8L); ~4.4 qt (Si 2.4L) |
| Oil filter | 15400-PLM-A02 · drain washer 94109-14000 (14 mm, replace each time) |
| Engine air filter | 17220-R1A-A01 |
| Cabin / pollen filter | 80292-SDA-407 (behind the glovebox, ~10 minutes) |
| CVT fluid | Honda HCF-2 ONLY — ~3 qt per drain and fill, every 25,000–30,000 mi |
| Manual trans fluid | Honda MTF — ~2 qt (Si / 5-speed) |
| Coolant | Honda Type 2 (blue) — never generic green or "universal" |
| Brake fluid | DOT 3 — every 3 years regardless of mileage |
| Spark plugs | Iridium, ~100,000 mi — do not re-gap |
| Battery | BCI Group 51R |
| Wipers | Sedan 26" / 22" · Coupe 28" / 24" |
| Tire pressure | Per the driver’s door jamb sticker (typically 32 psi) |
| Timing belt | None — this engine uses a chain, designed to last its life |
| Minder reset | Ignition on → show oil life → hold ~10 s until it blinks → hold ~5 s |
| Recall check | honda.com/recalls — free, forever, any dealer |

# The short version

- Follow the Maintenance Minder, not a mileage chart. Change the oil when the car asks, or once a year, whichever comes first.

- Use 0W-20 full synthetic. Not 5W-20, not 5W-30.

- Age items are almost certainly due: brake fluid every 3 years, coolant on the long interval. This car is 11.

- There is no timing belt. Anyone quoting one for this car is wrong.

- If it is a CVT, HCF-2 fluid every 25–30k is the single highest-leverage thing you can do.

- Run the VIN through honda.com/recalls today — Takata airbags touched this era.

- Check tire pressure monthly with a gauge; the indirect TPMS misses slow even leaks.

# Age-based items — what an 11-year-old car owes

Maintenance Minder measures engine and fluid condition. It does not track age. These do not care how little you drive.

| Item | Interval | Status at 11 years |
| --- | --- | --- |
| Brake fluid | Every 3 years, always | Should have been done 3 times. Absorbs water, lowers boil point, corrodes ABS parts internally. |
| Engine coolant | First ~10 years, then every 5 | Due now on age alone if never replaced. |
| Battery | 4–6 years | On its second or third. Load-test before winter. |
| Tires | 6–10 years regardless of tread | Check the DOT date code — 4 digits, week and year (3219 = week 32, 2019). |
| Drive belt | Inspect at each B service | Original at 11 years is on borrowed time. |
| Brake/fuel lines, bushings | Inspect | If the car has seen road salt, this is a rust inspection now. |

# Maintenance Minder codes

| Code | What it wants |
| --- | --- |
| A | Replace engine oil. |
| B | Replace engine oil and filter; inspect brakes, parking brake, tie rods, steering, suspension, driveshaft boots, brake and fuel lines, exhaust, all fluid levels. |
| 1 | Rotate tires; check pressure and condition. |
| 2 | Replace engine air cleaner and cabin filter; inspect drive belt. |
| 3 | Replace transmission fluid. |
| 4 | Replace spark plugs; inspect valve clearance and water pump. |
| 5 | Replace engine coolant. |
| 6 | Rear differential — never appears on this car (front-wheel drive). |

Codes combine: B12 = oil and filter, full inspection, rotate tires, both filters. One visit, one list.

Only reset what you actually did — resetting the main display clears every pending item at once.

# Mileage calendar

| Interval | Work |
| --- | --- |
| 6,000–10,000 mi | Oil and filter, tire rotation |
| ~15,000 mi | Add engine air filter and cabin filter |
| 25,000–30,000 mi | CVT fluid drain and fill — the interval that matters most |
| 30,000 mi | Brake inspection with pad measurement; clean and lube caliper slide pins |
| ~100,000 mi | Spark plugs; inspect valve clearance, water pump, drive belt |
| ~100,000–120,000 mi | First coolant replacement, then every 60,000 |
| Never | Timing belt — the engine uses a chain |

# DIY versus shop

Ballpark US independent-shop pricing, 2026. Dealers run 30–60% higher on maintenance, but recall work is free and correct at the dealer.

| Job | Shop price | Verdict |
| --- | --- | --- |
| Engine air filter | $40–70 | Do it yourself — two minutes, no tools |
| Cabin filter | $50–90 | Do it yourself — ten minutes, drop the glovebox |
| Wipers | $30–60 | Do it yourself |
| Battery | Free fit with purchase | Take the free fitting |
| Oil and filter | $70–110 | Shop — thin margin, messy disposal |
| Tire rotation | $25–50 | Shop — often free where you bought the tires |
| CVT drain and fill | $150–250 | Shop — and specify HCF-2 on the invoice |
| Brake fluid flush | $100–180 | Shop |
| Brake pads and rotors | $300–500 per axle | Shop unless experienced |
| Spark plugs | $150–250 | Either — torque to spec, aluminum head |
| Coolant replacement | $120–200 | Shop — must be burped of air |

Ask for your old parts back, or to see them. Costs nothing, and quietly ends the conversation about services charged but not performed.

# Known weak points

- CVT behaviour (2014–15 autos): shudder or hesitation at highway speed. Overwhelmingly a fluid-condition story — fresh HCF-2 every 25–30k. Judder that survives a fluid change wants a specialist.

- Takata airbag inflators: this era of Honda sat at the centre of the largest safety recall in US history. Free repair, no expiry.

- Repeat dead batteries are often a parasitic drain, not a bad battery. Have the draw measured before buying a third.

- Oil consumption on some engines of this era — check the level monthly rather than assuming.

- AC compressor and rear suspension bushings are normal wear items past 100k, not defects.

# Monthly five-minute walkaround

- Tire pressure, all four plus the spare, cold, with a real gauge.

- Tread depth — a quarter, Washington head down: see the top of his head and you are shopping for tires.

- Oil level on level ground, engine cool. Note level and colour.

- Coolant between MIN and MAX in the overflow tank, cold. Never open a hot cap.

- Brake fluid level — falling slowly means pads wearing, or a leak.

- Every light including brake lights — back up to a wall at night and watch the reflection.

- Listen once with the radio off: new ticking, grinding, droning, or a speed-dependent squeal.

# Service log

Keep this in the glovebox. A documented history is worth real money at resale and settles every “did I already do that?” question.

| Date | Mileage | Code | Work performed | Cost |
| --- | --- | --- | --- | --- |

## Caveats

The owner’s manual and the under-hood sticker are specific to your VIN and outrank this sheet wherever they disagree. Two places published sources genuinely vary: the coolant first interval is printed as 10 years / 100,000 miles in some manual revisions and 120,000 in others (academic at 11 years — it is due), and the 30,000-mile spark plug figure quoted all over the internet belongs to older conventional plugs, not the iridium plugs in this car. Prices are ballpark and move with region and labour rate. Nothing here replaces a qualified mechanic looking at the actual car.